10 Steps to Improve Usability, Utility, and Desirability by Implementing Nielsen and Molich’s UI Design Guidelines

- Choose the website or app for which you want to critique and improve usability, utility and desirability.

- Then work through the list to see whether or not the website or app follows Nielsen and Molich’s 10 rules of thumb.

- Finally, improve the website or app by further applying the 10 guidelines.

| Rule of Thumb | Is this rule being applied? How so? | Is this rule violated? How so? | How can this rule further improve usability, utility and desirability? |
| --- | --- | --- | --- |
| 1. Visibility of system status | Yes. The screen changes nearly instantly when a button is clicked. | No. | If drone information is only being updated periodically, then the time of the last update could be indicated. |
| 2. Match between system and the real world | Yes. The system uses terminology that the operator should be familiar with. The map can be moved like a map and the list is organized vertically like a list. | No. | “Task” could be replaced with “Current Destination” with appropriate adjustments. |
| 3. User control and freedom | Yes. There is a way to cancel every action that overrides the system. | No. | A “Cancel” button could be added to the Drone Control page. A “Back” button could be added to the Aggregate Fleet Statistics page. When vertically prototyped, Login, Issue Advisory, Register Drone, and Drone Recovery Request should have a way to undo an action if done accidentally. |
| 4. Consistency and standards | Yes. The same colours are used to represent each status. | Yes. “Current Leg” (in status) and “Task” (in list) mean the same thing. The default ordering of drones is different in the map and list. | “Pick up” (in status) and “Pickup” (in list) mean the same thing. “hr” (in aggregate fleet statistics), “hrs” (in list and history), and “h” (in history) are all used to abbreviate “hour(s)”. Disabled buttons look different in Register New Drone and status. The left column of the information boxes is right aligned in the weather summary and status, but left aligned in Aggregate Fleet Statistics and history. “Aggregate Fleet Statistics” (in aggregate fleet statistics) and “Fleet Statistics” (in menu) mean the same thing. “Register New Drone”/”Register” (in register new drone) and “Register Drone” (in menu) mean the same thing. “Recover Drone” (in status), “Drone Recovery Request”/”Submit Recovery Request” (in drone recovery request), and “Recovery Request” (in menu) mean the same thing. “Operate Drone” (in status), “Drone Control” (in drone control), and “Control” (in menu) mean the same thing. The symbol for headquarters is different in the map and the list. Numbers with at least four digits are formatted with spaces in Aggregate Fleet Statistics but formatted with commas in list and history. |
| 5. Error prevention | Yes. Buttons that do actions that are not applicable are disabled. | Yes. The operator cannot tell if there is a critical drone unless the operator is in the list with the critical filter on or in the map with both the list open and the critical filter on. | When vertically prototyped, Drone Recovery Request, Issue Advisory, and Register Drone should have confirmation messages. |
| 6. Recognition rather than recall | Yes. Drone Recovery Request, history, and status all show the relevant drone’s ID and status. | Yes. Drone Control does not show the relevant drone’s ID, status, and location. | Issue Advisory could include an option to view the weather summary. |
| 7. Flexibility and efficiency of use | Yes. The drone list allows an experienced operator to find a drone efficiently. The status and history of a drone can be accessed from both the map and list. | Yes. There should be keyboard shortcuts for common tasks. | There could be an option to scroll through the drone list instead of browsing pages. There could be a way to go directly from history to status. There could be a way to go directly from the history of one drone to the history of another drone. |
| 8. Aesthetic and minimalist design | Yes. The list only includes the most important pieces of information. The model information is hidden by default in the Register New Drone page. | No. | The “Map” and “Drone List” tabs could be made more visible than the other tabs. Pieces of information that are not applicable to a dormant drone could be hidden instead of displayed with a “~”. |
| 9. Help users recognize, diagnose and recover from errors | Yes. Critical drones have extra information with more details on the problem. | No. | When vertically prototyped, Login, Issue Advisory, Register New Drone, Drone Control, and Drone Recovery Request should have descriptive error messages. |
| 10. Help and documentation | Yes. Dropdown menus and text fields are labeled. Instructions for Drone Control are provided. The text field for serial number in Register New Drone has a placeholder indicating the length and format of the serial number. The status and task headers of the list can be hovered to see what the icons mean. | Yes. The map does not have a legend. It is not obvious that the status and task headers of the list can be hovered. | Forms in Issue Advisory, Register Drone, and Drone Recovery Request could indicate which fields are required and which are optional. |